# H30 2 LAN 4 COM Industrial PC

Features:

- Onboard with intel N2840/J2900 CPU;

- Support DDR3 SODIMM, Max Up to 8GB;

- 4*RS232 COM

- 2*RJ45 LAN Gigabit Ethernet;

- Fanless and low power consumption.

## Specification of Motherboard

| Model | H30 N2840/J2900 2 LAN 4 COM |
| --- | --- |
| Dimension | - 136*126*46mm - Motherboard size：121.6*121.6mm |
| Processor | - Intel® Celeron N2840/J2900 Processor |
| RAM | - 1*SO-DIMM DDR3L 204 Pin Memory Slot - Max support 8GB |
| Expansion | - 1 x MiniPCIe，support Wifi/4G Module |
| Front/Rear Interface | - 1 x DC-IN Power IN - 1 x HDMI - 1 x VGA - 5 x USB2.0 - 1 x USB3.0 - 2 x LAN - 1 x Audio - 2 x COM - 1 x HDD_LED - 1 x PWR Power Buttom |
| Inbuilt Interface | - 1 x SATA slot - 1 x SATA_PWR slot - 1 x GPIO Pin Optional COM3/4 Pin - 1 x F_PANEL - 1 x DC_IN Pin |
| BIOS | - AMI BIOS |
| Power Management | - Supports Advanced Power Management (ACPI) - Supports network wake-up for S3, S4, S5, and power-on startup, etc |
| Display | - VGA + HDMI, support synchronous/asynchronous dual display |
| Ethernet | - 2 x Realtek Gigabit LAN（RTL8111H） |
| Audio | - On-board ALC897 audio decoding controller Integrated power amplifier, 2W@4Ω |

| I/O Chipset | - IT8786E |
| --- | --- |
| Power | - DC/4Pin ATX Power supply |
| Working Environment | - Working Temperature：0℃~60℃ - Humidity：0%~95% |
| OS | Windows 7 / Windows 10/Linux |

- Motherboard Layout Diagram

The illustration is for reference only, please refer to the real thing.

| Ltem |  | Description |
| --- | --- | --- |
| 1 | JBAT1 | Inbuilt COMS Pin |
| 2 | SATA_PWR | SATA Power Slot |
| 3 | SATA | SATA Slot |
| 4 | GPIO | Inbuilt GPIO Slot |
| 5 | COM1/2/3/4 | Inbuilt COM Pin |
| 6 | JC11/21 | COM1/2 hot-plug capable select jumper |
| 7 | JC12/22 | COM RS485 select jumper |
| 8 | AT_SET | Auto-power-on select jumper |
| 9 | F_PANEL | Power Switch Pin |
| 10 | DCIN | Power Supply Pin |

| 11 | ATX1 | (Optional) 4-pin ATX power socket |
| --- | --- | --- |
| 12 | MSATA | mSATA slot |
| 13 | MPCIE | mPCIe slot |
| 14 | SODIMM1 | 1x SO-DIMM DDR3L memory slot |

## Interfaces

| Item | Description |
| --- | --- |
| DC_IN | 12V Power IN |
| VGA | Display Port(Optional HDMI) |
| HDMI1 | Display Port |
| LAN | Link LED: green long light, indicates network connected |
|  | Active LED: orange blinking, indicates data transmission |
| R_USB3_1 | USB2.0+USB3.0 |
| Audio | Audio Poer |
| COM1 | Serial Port 1 (configurable as COM header) |
| COM2 | Serial Port 2 (configurable as COM header) |
| F_USB2_2 | 2*USB2.0 |
| HDD_LED | HDD Activity LED |
| F_USB2_1 | 2*USB2.0 |
| PWRBTN1 | Power Button |